Lesson Planner

| Lesson plan for: Components Theory 1 |  |  | No: 5/12 | Date: 6/5/03 |
| --- | --- | --- | --- | --- |
| Year: L6th | Group: Revision | Room: Class 1 | Time/duration: 55min |  |

Summary of previous work and focus of this lesson Students have completed major project work and are now covering theory required for AS exam paper. This is to cover Unit 3 Section A (R301) Materials, Components and Systems over twelve lessons. This lesson is to introduce aspects of components used to join materials and translate motion or energy from one form to another.

Objectives Teaching Inputs: Pupils will learn about: Explain different nuts/bolts with thread types, theory of thread cutting and other joining methods (rivets) Explain gears, bushes and cams Introduce stock sizes to inform designing Learning Outcomes: By the end of the lesson pupils will be able to do or do better: Respond to Q&A during lesson to confirm understanding of components Work during prep to consolidate information and self test using text book tasks and practice exam questions on handouts

Links to: Literacy/Numeracy; Citizenship; SMSC development: SMSC – consider effect on society in choices by designers on whole of life issues – issue of reuse or ease of disassembly Citizenship – working together to revise content => co-operation

Resources required and risk assessment: Teacher Marker Board & Pens, Textbook Students Textbooks, notepads, pencils Risks Minimal in classroom environment

| Time/section | Activities, key teaching points, health and safety points |
| --- | --- |
| 5 min 15 min 5 min 5 min 5 min 5 min 10 min 5 min | Standard Operating Procedures –Take Register. Establish good order – No chewing, coats – shirts and ties Outline purpose of lesson on components to include nuts/bolts, thread cutting, rivets, gears, bushes and cams. Requires continuous feedback to ensure all topics are covered or recovered as necessary ready for exam Screw threads P 77 + P282-284 Three uses - Convert rotary to linear motion - Obtain mechanical advantage - Fastenings Mechanical advantage due to inclined plane Four types of thread - v-thread fasteners due to friction between flanks and nut ISO metric => dia. & pitch in mm (coarse or fine) older forms are BA, BSF, BSW, UNC & UNF - square moving parts in m/c/ lifting jacks – not as strong as vee less friction than vee but accepts large force - buttress force in one direction only e.g. woodworking vices/quick release - acme motion via engaging nut – tapered sides allow easy engagement of split nut on screw thread e.g. centre lathe External thread cutting using split die and die stock Internal thread cutting using tap and die Drunken thread is when die is not square to axis of rod turn 1 cut + ½ reverse to break swarf Taper rod to allow thread to start cutting Single point cutting also possible using center lathe Nuts and bolts Low to medium carbon steel – forged or machined threads my be cut or rolled May be galvanized to inhibit corrosion Alternative is coach bolt or stud. Use handout on nuts and bolts to illustrate common forms Rivets are a quick and convenient method of permanently fixing two or more pieces of material normally sheet metal. Cheap alternative to threaded fastenings which may be grouped to form rigid joints or act as a pivot/ hinge pin. Cams P273-276 Convert rotary to recriprocating motion via cam and follower Gears P78 + P158-160 See handout for different gear arrangements Toothed wheel with special profile for transmitting forces and motion 2+ gears = gear train with velocity ration proportional to no. of teeth idler gear used reverse or mainatain direction gears may achieve large speed reductions Bearings and bushes – radial & axial P79 Note lubrication – in special environments nylon or other plastics used in lieu of oil for plain, roller or ball bearings Bush is cylindrical sleeve fitted into hole as bearing surface => very high forces + use of sintered materials such as phosphor bronze with low friction plus gaps for lubrication (phosphor bronze is a ‘white metal’) Stock Sizes P79 Use handout to give pictorial view of stock sizes Designers need to note standard stock sizes e.g. sheet manuf. board 8’x4’ (2240x1220) or 4’x2’ (1220x610) Ceramics filler Glass, Concrete / Bricks, Tiles, sanitary ware Good heat, pressure and chemical resistance but brittle/hard & not ductile Use textbook on P76 to explain glass production on tin, plus ceramics made from clay, sand or feldspar (aluminum potassium silicate) Start to finish off ready for dismissal. Praise effort for work done and focus forward to reviewing industrial processes next lesson. Prep required – Answer questions on handouts Self check table on handout Identify revision needs and let teacher know any specific requirements Revise hand & commercial processes section Standard dismissal process, stand behind chairs, quiet and leave. |

Lesson Evaluation

| Date: | Group: | Time/duration: |
| --- | --- | --- |

Pupil response Sufficiency of subject content Appropriateness of teaching methods Achievement of learning outcomes Achievement of trainee targets

Trainee targets for next lesson

Follow up of individual pupils

Equipment/materials for next lesson

Materials,
Components and Systems

Fixings & Stock Shapes

Name

House

Date

Name

House

Materials,
Components and Systems

Gears and Threading

Date

Complete the following paragraph by inserting the correct terminology as required:

tap wrench, taper, plug, square, clockwise, split die, anticlockwise

Firstly, the correct sized hole should be drilled. The tap is then held in the _________. The ________   tap should be used to make the first cut followed by a _______ tap. The tap should be wound in slowly making sure it is _________ to the hole. Each _________ turn should be followed by a half turn in the opposite direction to break the swarf. If tapping a blind hole a _____ tap should be used. when threading a ____________ is held in the die stock. A taper should be filed on the bar being threaded to help the cutting process start. Dies are used in a similar way to the taps, a turn clockwise should be followed by a turn ___________.

Questions :

- Explain the main difference between a bolt and a machine or set screw

- What advantages does a wing nut have over a normal hexagonal nut