Lesson Planner

| Lesson plan for: Metal Theory 1 (revised) |  |  | No: 2/12 | Date: 28/4/03 |
| --- | --- | --- | --- | --- |
| Year: L6th | Group: Revision | Room: Class 1 | Time/duration: 55min |  |

Summary of previous work and focus of this lesson Students have completed major project work and are now covering theory required for AS exam paper. This is to cover Unit 3 Section A (R301) Materials, Components and Systems over twelve lessons. This lesson is to introduce properties of materials focusing on metals and alloys to explain terms, and explain microstructure and heat treatment in metals.

Objectives Teaching Inputs: Pupils will learn about: Review properties of metals to include composition of alloys, mechanical and physical properties Explain micro structure of metals using lattice structures and principles of grain structure Use microstructure to indicate effective heat treatments for altering metal properties Learning Outcomes: By the end of the lesson pupils will be able to do or do better: Respond to Q&A during lesson to confirm understanding of information and suggest reasons why particular process are appropriate. Work during prep to consolidate information and self test using text book task and practice exam questions on handouts

Links to: Literacy/Numeracy; Citizenship; SMSC development: SMSC – consider effect on society in choices by designers on whole of life issues Citizenship – working together to revise content => co-operation Chemistry – Chemical symbols and links re properties

Resources required and risk assessment: Teacher Marker Board & Pens, Textbook Students Textbooks, notepads, pencils Risks Minimal in classroom environment

| Time/section | Activities, key teaching points, health and safety points |
| --- | --- |
| 5 min 15 min 5 min 15min 10 min 5 min | Standard Operating Procedures –Take Register. Establish good order – No chewing, coats – shirts and ties Outline purpose of next four weeks is to review theory necessary for part A of module relating to materials, components and systems. Requires continuous feedback to ensure all topics are covered or recovered as necessary ready for exam – This lesson is material properties focused on metals Turn to pages 66 & 67 of textbook Three classes of metals – ferrous, non ferrous and alloys Material Properties P80 2 types – mechanical & physical Mechanical strength tensile, compressive, bending, shear, torsion hardness A measure of how good a material is at resisting being shaped or deformed when put under a force durability The ability of a material to withstand abrasive wear and indentation toughness A measure of how good a material is at resisting being shaped or deformed when put under a force elasticity The ability of a material to return to its original shape once a deforing force has been removed plasticity The ability of a material to be changed permanently without breaking or cracking ductility The property which allows a material to be drawn out, elongated or stretched malleability A material that can be deformed by compression without tearing or cracking Physical density The relationship between a material’s weight and volume corrosion resistance ability of a material to withstand weathering, deterioration and corrosion electrical resistance A material which has a low electrical resistance fusibility The ability to change into a liquid form or molten state at a certain temperature thermal properties A measure of how much heat travels through a material ability to reflect light The way in which a material reacts to light Elastic vs. plastic deformation example springs and deep drawing See page 93 for graph of stress vs strain Review Micro Structure of metals => CPH, FCC, BCC & 910°C Iron change from BCC to FCC + return to BCC above Crystallisation to form dendrites which lead to grains (boundaries visible under microscope) => Idea of grain structure Alloying – P79 Change melting point Increase strength, hardness and ductility Change electrical or thermal properties Enhance resistance to oxidation and corrosion Change colour / finish (e.g. brass and copper) Better casting properties Review table on page 68 and mini table page 80 for composition and properties Go through table step by step to ensure understanding Prep challenge – cover table and reproduce key properties Heat treatment and work hardening P81 Temperature and rate of cooling affect properties - Steel hardening only when carbon > 0.4% fully above 0.8% by heating to just above critical temperature (720°C for .8%C steel) + quench -> hard and brittle case hardening possible for low carbon steels by heating and placing in high carbon comp - Tempering involves reheating to a temperature which allows changes to properties, normally indicated by oxide colour on polished surface - Normalising used on forged components to obtain uniform grain structure by heating above critical temp followed by soak and still air cool - Work hardening happened during deformation of cold working – annealing required between process to allow further working - Annealing is the relief of internal stress through heat, grain regrowth is dependant on temperature and soak/cooling time Start to finish off ready for dismissal. Praise effort for work done and focus forward to reviewing thermoplastics next lesson. Prep required – Answer questions on handouts Self check table on p68 Identify needs from handout and let teacher know any specific requirements Revise plastics composition and processes Standard dismissal process, stand behind chairs, quiet and leave. |

Lesson Evaluation

| Date: | Group: | Time/duration: |
| --- | --- | --- |

Pupil response Sufficiency of subject content Appropriateness of teaching methods Achievement of learning outcomes Achievement of trainee targets

Trainee targets for next lesson

Follow up of individual pupils

Equipment/materials for next lesson

Name

House

Materials,
Components and Systems

Key Points to Revise 1/2

Date

Classification of materials and components

Sources, classification, formation and structure of materials and components:

Metals and alloys. Ferrous and non-ferrous metals

- Source and classification of metals with details of structures and manufacturing processes, ie the make-up of ferrous metals and the mixtures involved in common alloys.

- An understanding of such areas as conductivity, working properties, melting points and common uses of metals.

Thermoplastic and setting polymers

- The chemical make-up and polymer structure of plastics and the methods of working, ie GRP, injection and blow moulding, vacuum forming and fusion.

Hardwoods and softwoods, manufactured boards

- Characteristics and faults of woods, ie cupping, twisting, reasons for splitting, etc. Source of different types of wood. Conversion, seasoning and distribution.

Composites, synthetics and manufactured materials

- Manufacture of composite materials. Dangers of use, ie cutting and sanding MDF.

Ceramics, glass and concrete

- Sources of and manufacture of those materials and their industrial (in particular in the construction industry) applications.

Micro-structure of materials

Be aware of the micro-structure of materials:

- metals: crystals, grain and lattice structure

- plastics: monomers, polymerisation and cross-linking

- wood: cellular, fibres and grain direction

- specialist materials for specific tasks. For example: stainless steels, lightweight materials for aviation, special materials for the nuclear and chemical industries.

Classification of components

- Nuts/bolts - thread types and sizes and their various applications, together with the theory of thread cutting.

- Rivets - pop and snap (hot and cold).

- Gears - types and applications.

- Bushes and bearings and their uses.

- Cams - types and applications together with types of followers.

- Stock sizes.

Working properties of materials and components related to preparing, processing, manipulating and combining

Working properties and functions of materials and components, relating to the composition and structure of materials.

- Alloying metals to change properties.

- Mechanical properties of materials such as strength, elasticity, plasticity, ductility, hardness and malleability.

- Heat treatment, eg annealing.

- Work hardening, age hardening.

Name

House

Materials,
Components and Systems

Key Points to Revise 2/2

Date

Hand and commercial processes

Hand and commercial methods of preparing, processing, manipulating and combining materials and components to enhance their properties, including associated tools, machinery and equipment including CAD/CAM in relation to:

- fabrication/manufacture in one or more resistant materials

- processes - casting and sintering, shearing, stamping, milling, drilling, turning, grinding, spark erosion, laser cutting, forging

- removal - methods of cutting, abrading

- addition - techniques to include thermal, mechanical, chemical

- joining - permanent and semi-permanent, mechanical, fusion, adhesives.

Finishing techniques for function and decoration.

Applied finishes to improve quality and provide enhanced aesthetic or functional properties.

- Surface coating, for example - anodising, painting, varnishing, preservation methods.

- Self-finishing.

- Surface decoration, engraving, pyrographic methods of decorating wood, transfer techniques on plastic. Spray paints, etching, etc.

- The relationship between finishes, properties and quality.

Product manufacture, eg:

- one-off, batch or high-volume manufacture

- the impact of different levels of production on costs and production time.

Understand how one off/batch/high-volume production of products is achieved using:

- processes - casting and sintering, shearing, stamping, milling, drilling, turning, grinding, spark erosion, laser cutting, forging, metal folding, injection moulding

- removal - methods of cutting, abrading

- addition - techniques including thermal, mechanical, chemical

- joining - permanent and semi-permanent, mechanical, fusion, adhesives

- finishing - surface coating, applied finishes, self-finishing, surface decoration.

Testing Materials

The principles and techniques of testing before manufacture. Comparative testing of materials and components, processes and prototypes for quality, performance, safety, ease of manufacture, aftercare, maintenance and fitness-for-purpose:

Understand:

- the purpose of British and International Standards

- the need for safety testing under controlled conditions including strength and fire-testing requirements

- the use of information communication technology as a testing aid. For example: virtual reality, running test programmes before manufacturing to investigate the viability of machining

- ultrasound testing of materials.

Name

House

Materials,
Components and Systems

Properties of Materials

Date

Complete the following table by naming the property described and indicate wheather it is a mechanical or physical property. As an additional exercise, can you name a material and application for which each property is used to maximum effect?

|  | Property | Mech. or Physical? | Description |
| --- | --- | --- | --- |
| 1 |  |  | The way in which a material reacts to light |
| 2 |  |  | A measure of how good a material is at resisting being shaped or deformed when put under a force |
| 3 |  |  | The property which allows a material to be drawn out, elongated or stretched |
| 4 |  |  | A measure of how much heat travels through a material |
| 5 |  |  | The relationship between a material’s weight and volume |
| 6 |  |  | The ability of a material to return to its original shape once a deforing force has been removed |
| 7 |  |  | A material which has a low electrical resistance |
| 8 |  |  | The ability of a material to be changed permanently without breaking or cracking |
| 9 |  |  | The ability of a material to withstand abrasive wear and indentation |
| 10 |  |  | A material that can be deformed by compression without tearing or cracking |
| 11 |  |  | The ability to change into a liquid form or molten state at a certain temperature |
| 12 |  |  | The ability of a material to withstand weathering, deterioration and corrosion |

Questions …

1.	Name suitable destructive tests for the following properties and state why the tests are appropriate

- Tensile Strength

- Hardness

- Toughness

- Ductility

- Name at least three non-destructive types of testing that could be used to confirm that components meet a specification

- Research and list the stages required to extract ferrous and non ferrous metals from source mined ores to final refined pure metal.

Name

House

Materials,
Components and Systems

Heat Treatment

Date

The table below gives a definition of a heat treatment process. For each definition, give the correct heat treatment process.

|  | Process | Definition |
| --- | --- | --- |
| 1 |  | A piece of hardened steel is polished. It is heated to a particular colour and then quenched at once in oil |
| 2 |  | The metal is heated up to a set temperature and then allowed to cool in air |
| 3 |  | Mild steel is heated up to cherry red and then dipped into a carbon powder where it is left to cool. this process is repeated two or three times more before the metal is finally quenched in water |
| 4 |  | The components are heated up and soaked at this temperature before being allowed to cool in still air, sometimes in the oven which heated them |
| 5 |  | The piece of carbon steel is heated unitl cherry red and held at this temperature before being quenched in water |

Questions :

- Once a piece of steel has been hardened, it becomes brittle. Explain how to carry out the process to reduce this brittleness.

- Explain the following terms

  - Tempering

  - Normalising

  - Work Hardening

  - Annealing

  - Case Hardening

Name

House

Materials,
Components and Systems

Questions on Metals …

Date

Attempt the following practice questions:

  - Define the following terms

    - brittleness

    - malleability

    - tensile strength

    - elasticity

    - plasticity

    - ductility

    - hardness

  - Name a specific metal that is brittle

  - Name a specific metal that is malleable when cold

  - Give an example of an application that requires a malleable material

  - Explain what is meant by an alloy

  - Identify the metal content of mild steel

  - Give two benefits gained from making an alloy

  - Give one example where an alloy could be used

- The strength of a material is defined as the ability of a material to withstand forces without permenantly bending or breaking it. Define each of the five different types of strength and give an example of each.

- A piece of aluminium of 9mm diameter cracked when it was bent cold

  - State what caused this problem

  - Identify the process that could prevent this problem from occuring and explain its application

- Define the materials used in the following applications and explain how the properties and composition of each material contribute to the function of the product

  - Carving Knife

  - Cold Chisel

- Explain the heat treatment process for making a screwdriver that is both tough and hard wearing