Lesson Planner

| Lesson plan for: Metal Theory 1 (revised) |  |  | No: 2/12 | Date: 28/4/03 |
| --- | --- | --- | --- | --- |
| Year: L6th | Group: Revision | Room: Class 1 | Time/duration: 55min |  |

Summary of previous work and focus of this lesson Students have completed major project work and are now covering theory required for AS exam paper. This is to cover Unit 3 Section A (R301) Materials, Components and Systems over twelve lessons. This lesson is to introduce aspects of plastics structure and methods of working.

Objectives Teaching Inputs: Pupils will learn about: Explain difference in structure between thermoplastics and thermosetting Explain different processes used to work plastics into useful compounds Learning Outcomes: By the end of the lesson pupils will be able to do or do better: Respond to Q&A during lesson to confirm understanding of plastic chemical structures and effects on properties Work during prep to consolidate information and self test using text book tasks and practice exam questions on handouts

Links to: Literacy/Numeracy; Citizenship; SMSC development: SMSC – consider effect on society in choices by designers on whole of life issues Citizenship – working together to revise content => co-operation Chemistry – Chemical symbols and links re properties

Resources required and risk assessment: Teacher Marker Board & Pens, Textbook Students Textbooks, notepads, pencils Risks Minimal in classroom environment

| Time/section | Activities, key teaching points, health and safety points |
| --- | --- |
| 5 min 15 min 5 min 5 min 5 min 5 min 10 min 5 min | Standard Operating Procedures –Take Register. Establish good order – No chewing, coats – shirts and ties Outline purpose of next four weeks is to review theory necessary for part A of module relating to materials, components and systems. Requires continuous feedback to ensure all topics are covered or recovered as necessary ready for exam – This lesson is on plastics and working processes Turn to pages 67, 69 70 & 71 of textbook Two groups of plastic – thermoplastic and thermosetting + elastomers fall between two – rubber, flexible joints, etc. sketch cross links vs. van der waals bonding Difference is in way polymerization occurs. basic structure is chains of monomers which when grouped together exhibit plastic properties 1 ethylene = gas (monomer) 6 ethylene = liquid 36 ethylene = grease 140 ethylene = wax 500+ ethylene = plastic polyethylene (practical limit 2000, greater does not improve strength and may be more brittle/ hardness) Thermosetting normally by condensation as the plastic reacts with itself or catalyst/hardener under heat and pressure to condense out small molecules such as water resulting in a permanent change i.e. cured (loss of water => shrinkage) Thermoplastic and thermosetting plastics as handout Polythene Urea formaldehyde Polypropylene Melamine formaldehyde Polystyrene Polyester Resin (-> GRP) uPVC Epoxy Resin Acrylic Nylon Cellulose acetate ABS (Acrylonitrile butadiene styrene) Mainly self finished only – specialised painting available -> flexible coating Edge finishing prior to forming (bending) – flash in injection molding Plastic processes Thermoplastics supplied as homogeneous material. polymer, pigment, lubricant and release agent Thermosetting supplied as polymer bulked out with other fillers such as mica, aluminum, glass fiber, etc. to change properties and economics. Injection Molding Blow Molding Vacuum forming Fusion Welding – hot air gun/torch, ultrasound + pressure joins sheet Lamination Extrusion Injection molding – P70 High volume, high initial set up costs but very low component costs - automated Best suited to thermoplastics but not exclusive Heat material to plastic state Inject into closed mould under heat/pressure Allow solidification Open mould and eject molding using ejector pins Self finish, chop sprue pins, gates and runners for reuse (thermoplastics) Blow Molding -P70 +1.5 billion PET bottles (polyethylene teraphthalate)/annum High volume, high initial set up costs but very low component costs - automated extrude parison (hollow tube) of plastic close mould to seal end of tube inject air to expand to mould shape allow to cool, open and eject molding Vacuum forming – P71 Medium volume, component production cost higher – good for baths, packaging Simple shapes from thermoplastic ‘Normal’ acrylic plastic at 180°C – breakdown at 195°C heat sheet using radiant heaters blow sheet to stretch raise platen to insert mould into sheet extract air inside mould to allow air pressure to ‘force’ sheet onto mould cool and remove mould – rapidly due to shrinkage Fusion Welding – P71 hot air gun/torch + filler rod, ultrasound + pressure joins sheet Lamination – P69 + P82 GRP – Polyester resin + hardener/pigment as adhesive, glass fibers for strength Mould must be smooth and non porous – mould => tapered sides, no undercuts + release agent as plastics are also good adhesives, no sharp corners Excellent strength to weight ratio Disadvantages cutting composites => fine particles + resin inhalation/ skin irritant Extrusion Similar to injection molding however continuous process, feed screw provides continuous compression to extrude through die – e.g. uPVC window frames Wire coating a special case using cross head die with wire fed through middle of extrusion resulting in even coating Start to finish off ready for dismissal. Praise effort for work done and focus forward to reviewing wood next lesson. Prep required – Answer questions on handouts Self check table on handout Identify revision needs and let teacher know any specific requirements Revise wood section and processes Standard dismissal process, stand behind chairs, quiet and leave. |

Lesson Evaluation

| Date: | Group: | Time/duration: |
| --- | --- | --- |

Pupil response Sufficiency of subject content Appropriateness of teaching methods Achievement of learning outcomes Achievement of trainee targets

Trainee targets for next lesson

Follow up of individual pupils

Equipment/materials for next lesson

Name

House

Materials,
Components and Systems

Review of Plastics

Date

Complete the following table by naming the plastic choice from the initial list and expand with other examples as appropriate. The starter list is as follows

 low density polyethylene,  epoxy resin,  high density polyethylene,
 polyester resin,  ABS,  acrylic,  polystyrene

|  | Application | Plastic Choice and indicate thermoplastic (TP) or thermoset (TS) | Reason(s) for choice+ suitable method of manufacture. |
| --- | --- | --- | --- |
| 1 | Adhesives |  |  |
| 2 | Product cases such as food mixers or hair dryers |  |  |
| 3 | Baths and bathroom accessories |  |  |
| 4 | Buckets and bowls |  |  |
| 5 | Paper weight castings |  |  |
| 6 | Washing up liquid bottles |  |  |
| 7 | Disposable coffee cups |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |

Questions …

1.	Explain with the use of sketches the main stages in the production of a soft drinks bottle

2.	Injection moulding is widely used in the plastics manufacturing industry. Compare the set up costs in terms of machinery and the mould for with the unit cost of manufacture on production runs in excess of 100,000 products.

3.	List four means of transforming plastics into finished artifacts

4.	Explain the difference between thermo and thermosetting plastics and sketch the molecular structure.

5.	Describe two different means of joining plastic components

6.	Sketch the stages in injection moulding and vacuum forming and list the stages involves

Name

House

Materials,
Components and Systems

Common Plastics

Date

Thermoplastics

| Material | Properties and working characteristics | Uses |
| --- | --- | --- |
| Polythene (polyethylene) (LDPE) | Low density: tough, common plastic, good chemical resistance, flexible, soft, attracts dust, electrical insulator, wide range of colours | Detergent, squeezy bottles, toys, packaging film, carrier bags, TV cable |
| (HDPE) | High density: stiffer, harder, high softening point, can be sterilized, waxy feel | Milk crates, bottles, pipes, houseware, buckets, bowls |
| Polypropylene (PP) | Light, hard, impact resistant even at low temperatures, good chemical resistance, can be sterilised, easily joined, welded, good resistance to work-fatigue, bending, hinges, good mechanically | Medical equipment, syringes, containers with integral hinges, string, rope, nets, crates, chair shells, kitchenware, film |
| Polystyrene (PS) | a) Conventional: light, hard, stiff, colourless, transparent, brittle, low impact strength, safe with food, good water resistance | Model kits, packaging, disposable plates, cups, utensils,TV cabinets, containers |
|  | b) Toughened: increases impact, strength, pigmented | Toys, refrigerator linings |
|  | c) Expanded/foam: buoyant, lightweight, crumbles, good sound/heat insulator | Sound and heat insulation, packaging |
| uPVC (Polyvinyl chloride) Plasticised (PVC) | Good chemical, weather resistance, stiff, hard, tough, lightweight, wide colour ranges, needs to be stabilized for outdoor use Soft, flexible, good electrical insulator | Pipes, guttering, bottles, shoe soles, roofing sheets, records, window frames Underseal, hosepipes, wall coverings |
| Acrylic (Polymethyl methacrylate) | Stiff, hard, clear, very durable, IOX impact resistance of glass, but scratches easily, excellent light transmission, optic qualities, safe with food, good electrical insulator, colours well, easily machined, polishes well | Light units, illuminated signs, record player lids, aircraft canopies, windows, rear car lights/reflectors, furniture, sanitary ware |
| Nylon (Polyamide) | Creamy colour, hard, tough, resilient to wear, low coefficient of friction, bearing surfaces, self-lubricating, resistant to extremes of temperature, good chemical resistance, machines well, difficult to join except mechanically | Bearings, gear wheels, casings for power tools, curtain rail fittings, combs, clothing, stockings, hinges, filaments for brushes |
| Cellulose acetate | Tough, hard and stiff (can be made flexible), resilient, light in weight, transparent, non-flammable, easily machined, absorbs some moisture | Pen cases, photographic film, cutlery handles, knobs, lids, spectacle frames, containers |
| ABS (Acrylonitrile butadienestyrene) | High impact strength and toughness, scratch resistant, light and durable, good appearance, high surface finish, resistant to chemicals | Kitchen ware, cases for consumer durables (e.g. cameras), toys, safety helmets, car components, telephones, food processors/mixers |

Thermosetting plastics

| Material | Properties and working characteristics | Uses |
| --- | --- | --- |
| Urea formaldehyde (UF) | Stiff, hard, strong, brittle, heat resistant, good electrical insulator, wide range of light colours, adhesive (Aerolite) | (White) electrical fittings, domestic appliance parts (e.g. knobs), adhesives (wood), coating paper, textile |
| Melamine formaldehyde (MF) | Stiff, hard, strong, scratch resistant, low water absorption, odourless, stain resistant, resists some chemicals, wide range of colours | Tableware, decorative laminates for work surfaces, electrical insulation, buttons |
| Polyester resin (PR) | Stiff, hard, brittle (resilient when laminated GRP), good heat and chemical resistance, electrical insulator, resists ultra-violet light, good outdoors, contracts on curing, chair takes colour well | Casting, encapsulation, embedding, panels (with GRP), boats, car bodies, shells, containers |
| Epoxy resin (epoxide) (ER) | High strength when re-inforced, good chemical and wear resistance, resists heat to 250°C, electrical insulator, adhesive for bonding unlike materials, low shrinkage | Surface coatings, castings, encapsulation of electronic components, adhesives, laminating paper, PCB, tanks, pressure vessels |