Lesson Planner

| Lesson plan for: Wood Theory 1 |  |  | No: 4/12 | Date: 5/5/03 |
| --- | --- | --- | --- | --- |
| Year: L6th | Group: Revision | Room: Class 1 | Time/duration: 55min |  |

Summary of previous work and focus of this lesson Students have completed major project work and are now covering theory required for AS exam paper. This is to cover Unit 3 Section A (R301) Materials, Components and Systems over twelve lessons. This lesson is to introduce aspects of wood structure and methods of conversion and working.

Objectives Teaching Inputs: Pupils will learn about: Explain difference between hardwoods, softwoods and manufactured boards Explain processes of conversion and seasoning Learning Outcomes: By the end of the lesson pupils will be able to do or do better: Respond to Q&A during lesson to confirm understanding of wood structures Work during prep to consolidate information and self test using text book tasks and practice exam questions on handouts

Links to: Literacy/Numeracy; Citizenship; SMSC development: SMSC – consider effect on society in choices by designers on whole of life issues – issue of recycling and reuse for biological resources Citizenship – working together to revise content => co-operation

Resources required and risk assessment: Teacher Marker Board & Pens, Textbook Students Textbooks, notepads, pencils Risks Minimal in classroom environment

| Time/section | Activities, key teaching points, health and safety points |
| --- | --- |
| 5 min 15 min 10 min 10 min 10min 5 min | Standard Operating Procedures –Take Register. Establish good order – No chewing, coats – shirts and ties Outline purpose of lesson on wood properties, conversion and uses. Requires continuous feedback to ensure all topics are covered or recovered as necessary ready for exam Turn to pages 72, 73, 74 of textbook Wood is a natural composite with fibers in lignin (glue) Three groups of wood – softwood, hardwood and manufactured boards Sketch internal structure of wood hardwood – fibers (majority), vessels and parenchymas (silver flashes) oak, mahogany, beech softwood – tracheids(pits), parenchymas, resin canals (resin/gum waste) scots pine, red cedar, spruce manufactured – no grain structure heartwood -> strength and rigidity sap wood -> growth where food stored -> annual rings Natural product -> variable quality with knots, grain, warping & twisting, attack from insects and fungi. 8 Defects classed as follows: Shrinkage Warping – cupping across the width of board Bowing – cupping along the length of timber Twisting – a twist from side to side along length (torsion) Splits Logs split radially of not seasoned seasoned timber if dried out too quickly -> paint ends Knots Branch junctions weaken structure and may be ‘hard’ in softwoods Irregular grain Knots lead to changes in grain direction Shakes Heart & star caused by shrinkage Cut and ring due to strains in felling Thunder perpendicular air line cracks in African timber Fungal attack Decay in wood -> total loss of strength and weight Dry rot Thrives in damp conditions without air circulation reducing wood to powder and musty smell Wet rot Decomposition of wood due to alternating spongy wet and brittle dry Working properties affected by: - Weight - Texture - Durability - Colour - Odour - Ease of Working Conversion process from tree to wood ready for use Fell tree forest management required for sustainability normally during winter months during dormancy/ less sap & moisture Remove branches for storage/ transportation Local trees keep bark on to reduce shrinkage Imported trees are stripped of bark to remove insects Conversion Saw logs into commercial viable timber Slab, plain through and through Strips are cheap but prone to warping Used for small dia. trees Quarter/ radial sawn More wasteful and time consuming Better timber by making annual rings as short as poss. Seasoning Drying wood to remove excess water and bound water from cell walls At fiber saturation point (approx 30%) water is lost from cell walls resulting in shrinkage Equilibrium with surroundings => no more water loss Reducing below 20% desirable to improve strength, stability and Reduce decay 10% for interior use, Natural air seasoning uses atmospheric condition only however takes over 1 year for 25mm thickness Kiln dried in controlled conditions to achieve result Rough sawn or planed to desired size Rough sawn to nominal size Planed on two opposite sides = PBS Planed all round = PAR =< 3mm less than nominal size Manufactured boards - MDF – compressed tiny particles in glue – very stable - Plywood – veneers laminated together with grain at 90° between layers Always odd number for flat faces gives good stability - Blockboard – softwood strips glued with veneer top and bottom = strength - Chipboard – compress and glue small pieces of wood, cheap but hard to work - Hardboard – glued and compressed pulped wood, cheap substitute for ply Advantages - Stable and uniform - Thin sheets can be bent easily - Wide boards up to 1.5m wide, hardwood 300mm and softwood 200m max Disadvantages - Difficult to join - Adhesives blunt tools quickly - Edge finishing an issue - Thin sheets must be supported to retain shape Finishes - To prevent moisture absorption which could make wood unstable - Protect against decay or attack - Enhance appearance of product Preperation => smoothing with grain to flatten and eliminate scratches Stain colours and enhances grain, not protective Oil maintain natural appearance and some protection, easily renewed Polish beeswax replaced by modern synthetic finishes Varnishes plastic based eg polyurethane tough, matt, satin or gloss, clear or coloured Paint knotting applied to ensure resin doesn’t leak through Emulsion – water based not waterproof or durable Oil based – more expensive but waterproof and hard wearing Start to finish off ready for dismissal. Praise effort for work done and focus forward to reviewing component next lesson. Prep required – Answer questions on handouts Self check table on handout Identify revision needs and let teacher know any specific requirements Revise components section and processes Standard dismissal process, stand behind chairs, quiet and leave. |

Lesson Evaluation

| Date: | Group: | Time/duration: |
| --- | --- | --- |

Pupil response Sufficiency of subject content Appropriateness of teaching methods Achievement of learning outcomes Achievement of trainee targets

Trainee targets for next lesson

Follow up of individual pupils

Equipment/materials for next lesson

Materials,
Components and Systems

Review of Woods

Name

House

Date

Complete the following table by listing timbers by category. The starter list is as follows

 oak,  mahogany,  chipboard,  plywood,  pine,  blockboard,

 MDF,  ash,  beech,  birch,  hardboard

|  | Hardwoods | Manufactured Boards | Softwoods |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |
| 12 |  |  |  |

Questions …

1.	List the stages in the processing of wood from source to useful material.

2.	In what way is the moisture content of wood likely to cause a problem

3.	A manufacture of a wooden pull along childrens toy has a number of things to consider

a	What are the important selection criteria to consider appropriate to this product
b	Explain which type of solid wood could be suitable
c	Give a suitable finish for the product

4.	List 3 advantages and disadvantages of manufactued board vs. natural timber

5.	Name 5 defects that can occur in natural timber

6.	Identify and give reasons for two specific precautions to ensure personal safety when cutting and abrading MDF

Name

House

Materials,
Components and Systems

Application of Woods

Date

|  | Application | Timber Choice | Reason |
| --- | --- | --- | --- |
| 1 | Fire doors |  |  |
| 2 | Hockey sticks |  |  |
| 3 | Roof trusses/ constructional timbers |  |  |
| 4 | Cabinet backs and drawer bottoms |  |  |
| 5 | Children’s toys |  |  |
| 6 | Flat pack furniture |  |  |
| 7 | High quality garden furniture |  |  |
| 8 | Hoardings and shuttering for the construction industry |  |  |
| 9 | Window frames |  |  |
| 10 | Floor boards in a loft |  |  |
| 11 |  |  |  |
| 12 |  |  |  |

Materials,
Components and Systems

Common Woods

Name

House

Date

# Softwoods

| Name | Origin/Colour | Properties and working characteristics | Uses |
| --- | --- | --- | --- |
| Beech | Europe White to pinkish brown | Close grained, hard, tough and strong, works and finishes well but prone to warping | Functional furnature e.g. chairs, toys, tools, veneer, turned work, steam bending |
| Elm | Europe Light ressish brown | Tough, durable, cross grained which makes it difficult to work, does not split easily, has a tendancy to warp, good in water | Garden furnature after treatment, turning and furnature |
| Oak European English Japanese | Europe Light brown Japan Yellow brown | Very strong, heavy, durable, hard and tough, finishes well, open grained, it contains tannic acid which corrodes iron/steel fittings leaving a dark blue stainaing in the wood, expensive Slightly milder, easier to work but less durable | High class furnature, fittings, boat building, garden furnature, posts, veneer Interior woodwork and furnature |
| Ash | Europe Pale cream colour and light brown | Open grained, tough and flexible, good elastic qualities, works and finishes well | Tool handels, sports equipment, traditional coach building, ladders, laminating |
| Mahogany African (e.g. Sapele, utile) | Central-South America, West indies, west africa Pink reddish brown | Easy to work, fairly strong, medium weight, durable, available in long, wide boards, some difficult interlocking grain, prone to warping | Indoor furnature and shop fittings, panneling, veneers |
| Teak | Burma, India Golden brown | Hard, very strong and extremely durable, natural oils make it highly resistant to moisture, acids and alkalis, works easily but blunts tools quickly, darkens with exposure to light, very expensive | Quality furniture, outdoor furniture, boat building, laboratory equipment, turnery, veneers |
| Walnut African | Europe, USA, West Africa Yellow, brown, bronze, dark lines | Attractive, works well, durable, open cross-grained which makes planing and finishing difficult, available in large sizes | Furniture, gun stocks, furniture veneers |

## Hardwoods

| Name | Origin/Colour | Properties and working characteristics | Uses |
| --- | --- | --- | --- |
| Scots Pine (red deal) | N europe, russia Cream, Pale brown | Straight grained, but knotty, fairly strong, easy to work, cheap and readily available | Mainly constructional work, paints well, needs outdoor protection |
| Western Red Cedar | Canada, USA Dark reddish brown | Light in weight, knot free, soft, straight silky grain, natural oils make it durable against weather, insects and rot, easy to work, but weak and expensive | Iutdoor uses, timber cladding of external buildings, also wall panelling |
| Parana Pine | South America Pale yellow with red/brown streaks | Hard, straight grained, almost knot free, fairly strong and durable, smooth finish, tends to warp, expensive | Best quality interior joinery, e.g. staircases, built in furniture i.e. kitchens |
| Spruce (whitewood) | N Europe, America Creamy-white | Fairly strong, small hard knots, resistant to splitting, some resin pockets, not durable | General indoor work, whitewood furniture (e.g. kitchens) |